# Week 15: System Integration — Robot Arm Lamp

以下是你可以直接改编放到 Fab Academy 网页上的完整文档内容。

## Concept

My final project is a Robot Arm Lamp inspired by the Pixar Luxo Jr. (Le Lamp). It uses Feetech STS3215 serial bus servo motors to give the lamp articulated movement and a sense of "personality." The lamp can look around, nod, follow a person, and illuminate a workspace — all controlled by an ESP32-S3 microcontroller on a self-made PCB.

Reference: Pixar Luxo Jr.        My Project: Robot Arm Lamp
                                
     ╱▔╲                              ╱▔╲  ← LED + diffuser
    │ ◯ │ lamp head                   │💡│  STS3215 #3 (head tilt)
     ╲▁╱                              ╲▁╱
      │                                 │   upper arm
      │  spring arms                    │
     ╱                                 ╱    STS3215 #2 (elbow)
    ╱                                 ╱
   │                                 │      lower arm
   │                                 │
   ╰─┐                              ╰─┐    STS3215 #1 (shoulder)
  ┌───┴───┐                        ┌───┴───┐
  │ base  │                        │ base  │ ESP32-S3 PCB inside
  └───────┘                        └───────┘

## System Architecture

### Block Diagram

┌─────────────────────────────────────────────────────────────────┐
│                    ROBOT ARM LAMP — System Block Diagram         │
│                                                                 │
│  ┌──────────┐     ┌──────────────────────────────────────────┐  │
│  │ 7.4V DC  │────▶│            Power Distribution            │  │
│  │ Adapter  │     │  ┌─────────┐    ┌──────────┐            │  │
│  └──────────┘     │  │AMS1117  │    │  Direct   │            │  │
│                   │  │  3.3V   │    │   7.4V    │            │  │
│                   │  └────┬────┘    └─────┬─────┘            │  │
│                   └───────┼───────────────┼──────────────────┘  │
│                           │               │                     │
│                    ┌──────▼──────┐  ┌─────▼──────────────┐      │
│                    │  ESP32-S3   │  │  STS3215 Servo Bus │      │
│                    │  (Main MCU) │  │  ┌────┐┌────┐┌────┐│      │
│                    │             │  │  │ #1 ││ #2 ││ #3 ││      │
│  ┌──────────┐      │  UART1 TX───┼──▶│    ││    ││    ││      │
│  │ USB-C    │◀────▶│  UART1 RX◀──┼──│    ││    ││    ││      │
│  │(program) │      │             │  │  └────┘└────┘└────┘│      │
│  └──────────┘      │  GPIO4 ─────┼──┼──▶ LED Driver      │      │
│                    │  GPIO6 ◀────┼──┼─── Touch Pad       │      │
│                    │  GPIO8,9◀───┼──┼─── I2C Sensors     │      │
│                    │  WiFi ──────┼──┼──▶ Web Interface   │      │
│                    └─────────────┘  └────────────────────┘      │
└─────────────────────────────────────────────────────────────────┘

### Subsystem Map

| Subsystem | Components | Fab Academy Week |
| --- | --- | --- |
| Structure | 3D printed joints + arms, laser-cut base | Week 3, 5, 8 |
| Main PCB | ESP32-S3, power regulation, connectors | Week 6, 8 |
| Input | Touch sensor, VL53L1X distance sensor | Week 11 |
| Output | STS3215 ×3, LED module, MOSFET driver | Week 12 |
| Communication | STS3215 serial bus (half-duplex UART) + WiFi | Week 13 |
| Interface | Web-based control panel | Week 14 |
| Integration | Wiring, packaging, final assembly | Week 15 |

## Mechanical Integration

### Joint Layout (Le Lamp Structure)

The lamp has 3 degrees of freedom, each driven by one STS3215:

Side view (neutral position):

                    ╱═══╗
                   ╱ LED ║  ← Lamp Head (3D printed)
                  ╱══════╝
                 ╱
        ────── ● Joint 3: Head Tilt (STS3215 ID:3)
       │ Upper │
       │  Arm  │  120mm
       │       │
        ────── ● Joint 2: Elbow (STS3215 ID:2)
       │       │
       │ Lower │
       │  Arm  │  150mm
       │       │
        ────── ● Joint 1: Shoulder (STS3215 ID:1)
    ┌──────────────┐
    │     Base     │  Ø130mm, height 40mm
    │  (PCB inside)│
    │   + weight   │  ~300g ballast
    └──────────────┘

### 3D Printed Parts List

| Part | Material | Print Time | Notes |
| --- | --- | --- | --- |
| Base bottom | PLA | ~3h | 30% infill, slots for PCB + ballast |
| Base top cover | PLA | ~2h | Touch area + cable pass-through |
| Shoulder bracket | PLA | ~1.5h | U-shape, holds STS3215 #1 |
| Lower arm | PLA | ~2h | Hollow tube, 150mm, wall 3mm |
| Elbow bracket | PLA | ~1.5h | U-shape, holds STS3215 #2 |
| Upper arm | PLA | ~1.5h | Hollow tube, 120mm, wall 3mm |
| Head bracket | PLA | ~1h | Holds STS3215 #3 |
| Lamp shade | PLA (white) | ~2h | Conical, Ø80mm opening |
| Diffuser | Frosted acrylic 3mm | Laser cut | Press-fit into shade |

### Joint Detail (U-Bracket Design)

Each joint uses the same U-bracket principle to hold the STS3215:

Front view of joint bracket:
       
    ┌────┐         ┌────┐
    │    │         │    │
    │  ○ │ STS3215 │ ●──┼──▶ to arm (output shaft)
    │    │  inside  │    │
    │    │         │    │
    └────┴─────────┴────┘
         ▲
      base mounting holes
    
    ○ = bearing/bushing side
    ● = servo output shaft side

Inner width: 22mm (servo 20mm + 1mm clearance each side)
Wall thickness: 4mm
M3 screws through sidewalls into servo mounting holes

### Cable Routing

Cable path through the arm:

    Lamp Head ◄─── LED wires (2) + sensor wires (4)
        │
   ┌────┴────┐
   │ Joint 3 │ ◄── servo #3 cable (3 wires: VCC/DATA/GND)
   └────┬────┘     all cables pass through 8mm hole
        │
   ╔════╧════╗
   ║ Upper   ║  ← hollow interior, 8mm cable channel
   ║   Arm   ║     6 wires from head + 3 from servo #3
   ╚════╤════╝
        │
   ┌────┴────┐
   │ Joint 2 │ ◄── servo #2 cable joins (now 9+3=12 wires)
   └────┬────┘
        │
   ╔════╧════╗
   ║ Lower   ║  ← 12 wires total through this section
   ║   Arm   ║     BUT: STS3215 is daisy-chain!
   ╚════╤════╝     So only 3 wires run full length
        │          (VCC, DATA, GND)
   ┌────┴────┐     + 2 for LED + 4 for I2C sensor
   │ Joint 1 │     = 9 wires total
   └────┬────┘
        │
   ┌────┴─────────┐
   │   Base (PCB)  │
   └──────────────┘

Key insight: STS3215 daisy-chain means only 3 bus wires
run from base through all joints. Each servo taps in locally.

Actual wires from base to head:
  1. STS_VCC  (7.4V, shared bus) ─── red
  2. STS_DATA (serial bus)       ─── white
  3. STS_GND  (ground)           ─── black
  4. LED_VCC  (7.4V for LED)     ─── red (can share with #1)
  5. LED_GND  (LED return)       ─── black (can share with #3)
  6. I2C_VCC  (3.3V)             ─── orange
  7. I2C_SDA                     ─── yellow
  8. I2C_SCL                     ─── blue
  9. I2C_GND                     ─── black (can share with #3)

  Optimized: share VCC and GND → only 5 wires from base to head!
  + each servo taps into the 3-wire bus at its joint

Cable protection at joints:

- Silicone flexible wire (not stiff PVC)

- 15mm slack loop at each joint

- Software angle limits prevent over-rotation

- Heat-shrink tubing at joint entry/exit points

## Electronics Integration

### Main PCB

Board size: 60mm × 45mm (fits inside base)

         ┌──────────────────────────────────────────────┐
         │  ┌─────────┐   ┌───────┐    ┌───────┐       │
         │  │  DC IN   │   │STS 3P │    │I2C 4P │       │
         │  │  7.4V    │   │VCC/D/G│    │V/G/D/C│       │
         │  └────┬─────┘   └───┬───┘    └───┬───┘       │
         │       │             │            │            │
  USB-C ─┤  ┌───┴─────────────┴────────────┴────────┐   │
         │  │                                        │   │
         │  │          ESP32-S3-WROOM-1              │   │
         │  │        ▒▒▒▒▒ (antenna this end) ──────▶│   │ ← board edge
         │  │                                        │   │
         │  └───┬──────────┬──────────┬─────────────┘   │
         │      │          │          │                  │
         │  ┌───┴───┐  ┌──┴──┐  ┌───┴────┐             │
         │  │AMS1117│  │MOSFET│  │470µF   │             │
         │  │ 3.3V  │  │LED Q1│  │filter  │             │
         │  │+caps  │  │+Rs   │  │        │             │
         │  └───────┘  └─────┘  └────────┘             │
         │  ┌────┐  ┌────┐                              │
         │  │RST │  │BOOT│  LED indicator               │
         │  └────┘  └────┘  ●                           │
         └──────────────────────────────────────────────┘
              ▲ ▲ ▲ ▲
              M3 mounting holes (4 corners)

### Wiring Schematic (Key Circuit)

#### STS3215 Half-Duplex UART Connection

This is the most critical integration detail.
STS3215 uses half-duplex: TX and RX share ONE data wire.

ESP32-S3                          STS3215 Daisy Chain
                                
GPIO17 (TX) ───[100Ω]───┐       ┌──────┐  ┌──────┐  ┌──────┐
                         ├───────┤ #1   ├──┤ #2   ├──┤ #3   │
GPIO18 (RX) ─────────────┘       │Shlder│  │Elbow │  │Head  │
                         DATA    └──┬───┘  └──┬───┘  └──┬───┘
                                    │VCC      │VCC      │VCC
7.4V ──┬─[470µF]──GND              │         │         │
       │                            │         │         │
       └────────────────────────────┴─────────┴─────────┘

The 100Ω resistor on TX prevents bus contention when
the servo is transmitting feedback data back to ESP32.

#### LED Driver

GPIO4 ──[1kΩ]──┬── Gate (AO3400A N-MOSFET)
               │
           [10kΩ]     Drain ── LED module (–)
               │       │
              GND    Source    LED module (+) ── 7.4V
                       │
                      GND

PWM on GPIO4 → variable brightness (0-255)

### Power Budget

Component             Voltage   Current (max)   Power
─────────────────────────────────────────────────────
ESP32-S3 (WiFi on)    3.3V      350mA           1.2W
STS3215 × 3 (moving)  7.4V      500mA each      11.1W
STS3215 × 3 (idle)    7.4V      100mA each      2.2W
LED module (full)      7.4V      350mA           2.6W
Sensors               3.3V      20mA            0.1W
─────────────────────────────────────────────────────
Total (peak):                    ~2.7A @ 7.4V    ~17W
Total (typical):                 ~1.0A @ 7.4V    ~7W

Power supply chosen: 7.4V 3A DC adapter (>20W headroom)
Bulk capacitor: 470µF/16V (absorb servo current spikes)

## Software Integration

### Architecture

┌──────────────────────────────────────────────────┐
│                  Software Layers                  │
├──────────────────────────────────────────────────┤
│                                                  │
│  Layer 4: User Interface                         │
│  ┌────────────────────────────────────────────┐  │
│  │  Web Page (HTML/CSS/JS)                    │  │
│  │  - Sliders for joint control               │  │
│  │  - Preset pose buttons                     │  │
│  │  - Animation triggers                      │  │
│  │  - Status display                          │  │
│  └────────────────────────────────────────────┘  │
│                                                  │
│  Layer 3: Behavior / Animation                   │
│  ┌────────────────────────────────────────────┐  │
│  │  State Machine:                            │  │
│  │  IDLE → CURIOUS → WORKING → SLEEPING      │  │
│  │  Animations: wake, nod, dance, greet       │  │
│  │  Idle micro-motions (breathing, sway)      │  │
│  └────────────────────────────────────────────┘  │
│                                                  │
│  Layer 2: Device Drivers                         │
│  ┌──────────┐ ┌────────┐ ┌─────────┐ ┌──────┐  │
│  │ SCServo  │ │LED PWM │ │ VL53L1X │ │Touch │  │
│  │ Library  │ │ Driver │ │ Driver  │ │Read  │  │
│  └──────────┘ └────────┘ └─────────┘ └──────┘  │
│                                                  │
│  Layer 1: Hardware Abstraction                   │
│  ┌──────────┐ ┌────────┐ ┌─────────┐            │
│  │ UART1    │ │ LEDC   │ │  I2C    │            │
│  │ 1Mbps   │ │ 5kHz   │ │ 400kHz  │            │
│  └──────────┘ └────────┘ └─────────┘            │
│                                                  │
└──────────────────────────────────────────────────┘

### Main Firmware (Complete Integrated Code)

// ============================================
// Robot Arm Lamp — Final Integrated Firmware
// Fab Academy 2024 — Final Project
// ============================================

#include <SCServo.h>
#include <WiFi.h>
#include <WebServer.h>
#include <Wire.h>

// ============ PIN DEFINITIONS ============
#define STS_RX    18
#define STS_TX    17
#define LED_PIN   4
#define TOUCH_PIN 6
#define I2C_SDA   8
#define I2C_SCL   9

// ============ SERVO IDs ============
#define ID_SHOULDER 1
#define ID_ELBOW    2
#define ID_HEAD     3

// ============ OBJECTS ============
SMS_STS sms_sts;
WebServer server(80);

// ============ POSE STRUCTURE ============
struct Pose {
  int shoulder;
  int elbow;
  int head;
  int led;  // 0-255, -1 = no change
};

// ============ PRESET POSES ============
// All values 0-4095, center = 2048
// Adjust these after mechanical assembly!
Pose POSE_HOME       = {2048, 2048, 2048, 100};
Pose POSE_SLEEP      = {2048, 2700, 2900,   0};
Pose POSE_WORK       = {2048, 2400, 2700, 255};
Pose POSE_LOOK_UP    = {2048, 1600, 1400,  60};
Pose POSE_CURIOUS_L  = {1700, 1900, 2200, 100};
Pose POSE_CURIOUS_R  = {2400, 1900, 2200, 100};
Pose POSE_NOD_DOWN   = {2048, 2048, 2300, 100};

// ============ STATE MACHINE ============
enum State { IDLE, SLEEPING, WORKING, CURIOUS, GREETING };
State currentState = IDLE;
unsigned long stateTime = 0;
unsigned long lastAction = 0;

// ============ SETUP ============
void setup() {
  Serial.begin(115200);
  Serial.println("\n=== Robot Arm Lamp ===");

  // Servo bus
  Serial1.begin(1000000, SERIAL_8N1, STS_RX, STS_TX);
  sms_sts.pSerial = &Serial1;

  // LED
  ledcSetup(0, 5000, 8);
  ledcAttachPin(LED_PIN, 0);
  ledcWrite(0, 0);

  // I2C
  Wire.begin(I2C_SDA, I2C_SCL);

  // Check servos
  Serial.println("Scanning servos...");
  for (int id = 1; id <= 3; id++) {
    if (sms_sts.Ping(id) != -1) {
      Serial.printf("  Servo %d: OK\n", id);
    } else {
      Serial.printf("  Servo %d: NOT FOUND\n", id);
    }
  }

  // WiFi AP
  WiFi.softAP("RobotLamp", "12345678");
  Serial.printf("WiFi AP: RobotLamp @ %s\n",
                WiFi.softAPIP().toString().c_str());

  // Web routes
  server.on("/", handleRoot);
  server.on("/move", HTTP_POST, handleMove);
  server.on("/pose", HTTP_POST, handlePose);
  server.on("/led",  HTTP_POST, handleLED);
  server.on("/anim", HTTP_POST, handleAnim);
  server.begin();

  // Wake up animation
  wakeUp();
  currentState = IDLE;
  stateTime = millis();
}

// ============ MAIN LOOP ============
void loop() {
  server.handleClient();

  unsigned long now = millis();

  switch (currentState) {
    case IDLE:
      // Breathing LED
      {
        float t = now / 1000.0;
        int b = 80 + sin(t * 1.2) * 20;
        ledcWrite(0, b);
      }
      // Random micro-movement every 3-6 seconds
      if (now - lastAction > random(3000, 6000)) {
        int r = random(0, 3);
        if (r == 0) microSway();
        else if (r == 1) microNod();
        else glanceSide();
        lastAction = now;
      }
      break;

    case SLEEPING:
      {
        float t = now / 1000.0;
        int b = max(0.0, sin(t * 0.5) * 15.0);
        ledcWrite(0, b);
      }
      break;

    case WORKING:
      ledcWrite(0, 255);
      break;

    case CURIOUS:
    case GREETING:
      break;
  }

  // Touch detection
  int touchVal = touchRead(TOUCH_PIN);
  if (touchVal < 25) {  // calibrate this threshold!
    Serial.println("Touch!");
    if (currentState == SLEEPING) { wakeUp(); currentState = IDLE; }
    else if (currentState == IDLE) { greet(); }
    else if (currentState == WORKING) { dance(); currentState = IDLE; }
    delay(500);  // debounce
  }

  delay(20);
}

// ============ MOTION FUNCTIONS ============

void moveTo(Pose p, int ms) {
  byte ids[3] = {ID_SHOULDER, ID_ELBOW, ID_HEAD};
  s16  pos[3] = {(s16)p.shoulder, (s16)p.elbow, (s16)p.head};
  u16  spd[3] = {0, 0, 0};
  byte acc[3] = {50, 50, 50};
  sms_sts.SyncWritePosEx(ids, 3, pos, spd, acc);
  if (p.led >= 0) ledcWrite(0, p.led);
  delay(ms);
}

void moveOne(byte id, int pos, int speed, byte acc) {
  sms_sts.WritePosEx(id, pos, speed, acc);
}

// ============ ANIMATIONS ============

void wakeUp() {
  Serial.println("Animation: Wake Up");
  moveTo(POSE_SLEEP, 0);          // start curled
  delay(500);

  Pose stretch = {2048, 2400, 2500, 0};
  moveTo(stretch, 1500);           // slowly uncurl

  for (int i = 0; i <= 100; i += 2) {  // LED fade in
    ledcWrite(0, i);
    delay(15);
  }

  moveTo(POSE_HOME, 1000);        // stand up
  delay(300);

  nod(1);                          // ready nod
}

void nod(int times) {
  for (int i = 0; i < times; i++) {
    moveTo(POSE_NOD_DOWN, 250);
    Pose up = POSE_HOME;
    up.head = 1900;
    moveTo(up, 200);
    moveTo(POSE_HOME, 300);
  }
}

void greet() {
  Serial.println("Animation: Greet");
  Pose excited = {2048, 1600, 1700, 200};
  moveTo(excited, 300);
  delay(200);
  nod(2);
  moveTo(POSE_HOME, 800);
}

void dance() {
  Serial.println("Animation: Dance");
  unsigned long start = millis();
  while (millis() - start < 4000) {
    float t = (millis() - start) / 1000.0;
    int s = 2048 + sin(t * 3.0) * 200;
    int e = 2048 + sin(t * 2.0 + 1.0) * 150;
    int h = 2048 + cos(t * 4.0) * 100;
    int l = (sin(t * 6.0) + 1) * 127;
    moveOne(ID_SHOULDER, s, 0, 10);
    moveOne(ID_ELBOW, e, 0, 10);
    moveOne(ID_HEAD, h, 0, 10);
    ledcWrite(0, l);
    delay(20);
  }
  moveTo(POSE_HOME, 1000);
}

void microSway() {
  int offset = random(-40, 40);
  Pose p = POSE_HOME;
  p.shoulder += offset;
  p.head -= offset / 2;
  moveTo(p, 1500);
}

void microNod() {
  Pose p = POSE_HOME;
  p.head += random(30, 80);
  moveTo(p, 600);
  delay(200);
  moveTo(POSE_HOME, 800);
}

void glanceSide() {
  int dir = random(0, 2) ? 1 : -1;
  Pose p = POSE_HOME;
  p.shoulder += dir * random(100, 200);
  moveTo(p, 700);
  delay(600);
  moveTo(POSE_HOME, 1000);
}

// ============ WEB HANDLERS ============

void handleMove() {
  if (server.hasArg("s") && server.hasArg("e") && server.hasArg("h")) {
    Pose p = {
      server.arg("s").toInt(),
      server.arg("e").toInt(),
      server.arg("h").toInt(),
      -1
    };
    moveTo(p, server.hasArg("t") ? server.arg("t").toInt() : 500);
    server.send(200, "text/plain", "OK");
  }
}

void handlePose() {
  String name = server.arg("name");
  if      (name == "home")  moveTo(POSE_HOME, 1000);
  else if (name == "work")  { moveTo(POSE_WORK, 1500); currentState = WORKING; }
  else if (name == "sleep") { moveTo(POSE_SLEEP, 2000); currentState = SLEEPING; }
  else if (name == "look")  moveTo(POSE_LOOK_UP, 1000);
  server.send(200, "text/plain", "OK");
}

void handleLED() {
  int b = server.arg("b").toInt();
  ledcWrite(0, constrain(b, 0, 255));
  server.send(200, "text/plain", "OK");
}

void handleAnim() {
  String name = server.arg("name");
  if      (name == "wake")  wakeUp();
  else if (name == "nod")   nod(2);
  else if (name == "dance") dance();
  else if (name == "greet") greet();
  currentState = IDLE;
  server.send(200, "text/plain", "OK");
}

// ============ WEB PAGE ============

void handleRoot() {
  String html = R"rawliteral(
<!DOCTYPE html>
<html><head>
<meta name="viewport" content="width=device-width,initial-scale=1">
<title>Robot Arm Lamp</title>
<style>
  *{box-sizing:border-box;margin:0;padding:0}
  body{font-family:sans-serif;background:#1a1a2e;color:#eee;padding:16px;max-width:420px;margin:0 auto}
  h1{text-align:center;font-size:1.4em;margin:12px 0}
  .c{background:#16213e;border-radius:10px;padding:14px;margin-bottom:12px}
  .c h2{font-size:.95em;color:#ffd700;margin-bottom:10px}
  .s{margin-bottom:10px}
  .s label{display:flex;justify-content:space-between;font-size:.85em;margin-bottom:3px}
  input[type=range]{width:100%;height:5px;-webkit-appearance:none;background:#0f3460;border-radius:3px}
  input[type=range]::-webkit-slider-thumb{-webkit-appearance:none;width:18px;height:18px;background:#ffd700;border-radius:50%}
  .g{display:grid;grid-template-columns:1fr 1fr;gap:6px}
  .b{padding:10px;border:none;border-radius:6px;font-size:.85em;font-weight:bold;cursor:pointer}
  .bp{background:#0f3460;color:#fff}.ba{background:#533483;color:#fff}
</style></head><body>
<h1>💡 Robot Arm Lamp</h1>
<div class="c"><h2>Joint Control</h2>
  <div class="s"><label>Shoulder <span id="vs">2048</span></label>
  <input type="range" id="rs" min="1024" max="3072" value="2048" oninput="vs.textContent=this.value;send()"></div>
  <div class="s"><label>Elbow <span id="ve">2048</span></label>
  <input type="range" id="re" min="1200" max="3200" value="2048" oninput="ve.textContent=this.value;send()"></div>
  <div class="s"><label>Head <span id="vh">2048</span></label>
  <input type="range" id="rh" min="1200" max="2800" value="2048" oninput="vh.textContent=this.value;send()"></div>
  <div class="s"><label>LED <span id="vl">100</span></label>
  <input type="range" id="rl" min="0" max="255" value="100" oninput="vl.textContent=this.value;fetch('/led',{method:'POST',headers:{'Content-Type':'application/x-www-form-urlencoded'},body:'b='+this.value})"></div>
</div>
<div class="c"><h2>Poses</h2><div class="g">
  <button class="b bp" onclick="pose('home')">🏠 Home</button>
  <button class="b bp" onclick="pose('work')">💡 Work</button>
  <button class="b bp" onclick="pose('sleep')">😴 Sleep</button>
  <button class="b bp" onclick="pose('look')">👆 Look Up</button>
</div></div>
<div class="c"><h2>Animations</h2><div class="g">
  <button class="b ba" onclick="anim('wake')">☀️ Wake</button>
  <button class="b ba" onclick="anim('nod')">👋 Nod</button>
  <button class="b ba" onclick="anim('dance')">💃 Dance</button>
  <button class="b ba" onclick="anim('greet')">🎉 Greet</button>
</div></div>
<script>
let T;
function send(){clearTimeout(T);T=setTimeout(()=>{
  fetch('/move',{method:'POST',headers:{'Content-Type':'application/x-www-form-urlencoded'},
  body:'s='+rs.value+'&e='+re.value+'&h='+rh.value+'&t=300'})},80)}
function pose(n){fetch('/pose',{method:'POST',headers:{'Content-Type':'application/x-www-form-urlencoded'},body:'name='+n})}
function anim(n){fetch('/anim',{method:'POST',headers:{'Content-Type':'application/x-www-form-urlencoded'},body:'name='+n})}
</script></body></html>
)rawliteral";
  server.send(200, "text/html", html);
}

## Packaging

### Design Philosophy

The goal is to make the lamp look like a finished consumer product, not a prototype. This means:

1. No visible wires on the outside
2. No visible PCB or solder joints
3. Clean joint transitions
4. Consistent color and finish
5. Stable base (won't tip over)
6. Easy access for maintenance (screw-off base cover)

### Packaging Layers

Layer 1 — Functional (hidden inside)
├── PCB mounted on brass standoffs inside base
├── Servos press-fit + screwed into U-brackets
├── LED module bolted to aluminum heat spreader in head
└── All solder joints covered by heat-shrink tubing

Layer 2 — Structural
├── 3D printed shells (base, arms, head) enclose everything
├── Cables routed through hollow arm interiors
├── M3 fasteners hold structural joints
└── Ballast weight (steel plate) in base for stability

Layer 3 — Finish
├── Sanded surfaces (220 → 400 grit)
├── Consistent white/gray color (white PLA or spray paint)
├── Frosted acrylic diffuser on lamp head (soft light output)
├── Silicone anti-slip pads on base bottom
└── Fab Academy label/logo on back of base

Layer 4 — Interaction
├── Touch-sensitive area on top of base (copper foil under 1mm cover)
├── USB-C port accessible at rear of base (for reprogramming)
├── Power jack at rear of base (clean entry with strain relief)
└── Small status LED visible through thin wall of base

### Base Interior Layout

Top view (base cover removed):

    ┌──────────────────────────────────┐
    │                                  │
    │   ┌─────────────────────┐        │
    │   │     PCB (60×45mm)   │        │
    │   │   ┌─────────────┐   │        │
    │   │   │  ESP32-S3   │   │        │
    │   │   └─────────────┘   │        │
    │   │  [LDO] [MOSFET]    │        │
    │   │  [caps]             │        │
    │   └─────────┬───────────┘        │
    │             │ cables up           │
    │    ┌────────┴────────┐           │
    │    │  Shoulder servo  │           │
    │    │  bracket (top)   │           │
    │    └─────────────────┘           │
    │                                  │
    │  ██████████  ← steel ballast     │ 
    │  ██████████    plate (300g)      │
    │                                  │
    │           [DC] [USB-C] ← rear    │
    └──────────────────────────────────┘

### Lamp Head Detail

Cross section:

      ┌──┐ ← connection to servo #3
      │  │    via shaft coupler
    ╱ └──┘ ╲
   ╱        ╲
  ╱  ┌────┐  ╲   ← 3D printed shade (white PLA)
 ╱   │ AL │   ╲      conical, wall 2mm
│    │plate│    │
│    │+LED │    │  ← aluminum plate for heat spreading
│    │ 5W  │    │     LED module screwed to it
│    └────┘    │
│              │
│ ═══════════ │  ← frosted acrylic diffuser
│  press-fit   │     press-fit into groove in shade rim
└──────────────┘
    Ø 80mm opening

## Assembly Process

### Step-by-step Integration

Step 1: Electronics Preparation
┌─────────────────────────────────────────────────────┐
│ □ Solder main PCB                                    │
│ □ Flash firmware via USB-C                           │
│ □ Test ESP32 boots, WiFi AP appears                  │
│ □ Connect ONE servo → set ID to 1                    │
│ □ Disconnect, connect next servo → set ID to 2       │
│ □ Repeat for servo 3                                 │
│ □ Daisy-chain all 3 → verify Ping all IDs            │
│ □ Test basic motion (each servo one by one)           │
│ □ Test SyncWrite (all 3 simultaneously)              │
│ □ Test LED PWM                                       │
└─────────────────────────────────────────────────────┘

Step 2: Mechanical Assembly (bottom → up)
┌─────────────────────────────────────────────────────┐
│ □ Mount PCB in base with M3×6 + brass standoffs      │
│ □ Place ballast weight in base                       │
│ □ Mount servo #1 in shoulder bracket                 │
│ □ Fix shoulder bracket to base                       │
│ □ Attach lower arm to servo #1 output shaft          │
│   (use servo horn + M2 screws)                       │
│ □ Mount servo #2 in elbow bracket at top of lower arm│
│ □ Attach upper arm to servo #2 output shaft          │
│ □ Mount servo #3 in head bracket at top of upper arm │
│ □ Attach lamp head to servo #3 output shaft          │
└─────────────────────────────────────────────────────┘

Step 3: Wiring
┌─────────────────────────────────────────────────────┐
│ □ Connect servo daisy chain cables                   │
│ □ Route bus wire through arm interiors               │
│ □ Leave slack loops at each joint (~15mm)            │
│ □ Run LED power wires to lamp head                   │
│ □ Connect LED module in head                         │
│ □ Tidy wires in base with zip ties                   │
│ □ Test: power on, move all joints, check no snag     │
└─────────────────────────────────────────────────────┘

Step 4: Calibration
┌─────────────────────────────────────────────────────┐
│ □ Physically position lamp straight up               │
│ □ Read all servo positions → these are your HOME     │
│ □ Update POSE_HOME values in code                    │
│ □ Move each joint to mechanical limits               │
│ □ Set software angle limits (prevent collision)      │
│ □ Test all animations with real structure             │
│ □ Adjust Pose values until motions look natural      │
└─────────────────────────────────────────────────────┘

Step 5: Packaging
┌─────────────────────────────────────────────────────┐
│ □ Sand 3D printed parts (220 → 400 grit)            │
│ □ Snap/screw base top cover                          │
│ □ Press-fit acrylic diffuser into lamp head          │
│ □ Apply silicone pads to base bottom                 │
│ □ Strain relief on DC power cable                    │
│ □ Final appearance check: no gaps, no wires visible  │
└─────────────────────────────────────────────────────┘

Step 6: Final Test
┌─────────────────────────────────────────────────────┐
│ □ Cold start: plug in, watch wake-up animation       │
│ □ Touch: lamp responds                               │
│ □ WiFi: connect phone, open web UI, control works    │
│ □ All poses reachable without collision               │
│ □ LED illuminates work area                          │
│ □ Idle motion looks natural                          │
│ □ Running for 30 min: no overheating                 │
└─────────────────────────────────────────────────────┘

## Bill of Materials

| # | Component | Spec | Qty | Source | Unit $ | Total $ |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Servo motor | Feetech STS3215 | 3 | AliExpress | 12 | 36 |
| 2 | MCU module | ESP32-S3-WROOM-1 | 1 | LCSC | 3.5 | 3.5 |
| 3 | LDO | AMS1117-3.3 SOT-223 | 1 | LCSC | 0.2 | 0.2 |
| 4 | MOSFET | AO3400A SOT-23 | 1 | LCSC | 0.1 | 0.1 |
| 5 | Capacitors | 100nF, 10µF, 22µF, 470µF | 5 | LCSC | — | 0.5 |
| 6 | Resistors | 100Ω, 1kΩ, 4.7kΩ, 10kΩ (0603) | 6 | LCSC | — | 0.3 |
| 7 | USB-C connector | 16P SMD | 1 | LCSC | 0.3 | 0.3 |
| 8 | Pin headers | 1×3, 1×4 | 3 | LCSC | — | 0.2 |
| 9 | Tactile switch | 6×6mm SMD | 2 | LCSC | 0.05 | 0.1 |
| 10 | LED | 5W warm white COB | 1 | AliExpress | 1 | 1 |
| 11 | Distance sensor | VL53L1X breakout | 1 | AliExpress | 3 | 3 |
| 12 | Copper foil tape | 20×20mm (touch pad) | 1 | Local | 0.5 | 0.5 |
| 13 | PCB (FR4) | 60×45mm, self-milled | 1 | Fab Lab | — | — |
| 14 | PLA filament | White, 1.75mm | ~120g | Fab Lab | — | 3 |
| 15 | Acrylic | Frosted 3mm | 1pc | Fab Lab | — | 1 |
| 16 | Screws | M3×6, M3×10, M2×6 | 30+ | Hardware | — | 2 |
| 17 | Brass standoffs | M3×8 | 4 | Hardware | — | 1 |
| 18 | DC adapter | 7.4V 3A | 1 | AliExpress | 5 | 5 |
| 19 | Silicone wire | 24AWG, 5 colors | 2m | AliExpress | 1 | 1 |
| 20 | Heat shrink | Assorted | — | Fab Lab | — | 0.5 |
| 21 | Steel plate | ~100×80×3mm (ballast) | 1 | Hardware | 2 | 2 |
| 22 | Silicone feet | Ø10mm adhesive | 4 | Hardware | — | 0.5 |
|  |  |  |  |  | Total | ~$62 |

## Challenges & Solutions

| # | Problem | Solution |
| --- | --- | --- |
| 1 | Servo #2 not responding after daisy-chain | GND wire was loose at connector — re-soldered with better joint |
| 2 | Arm hits base when shoulder goes too far | Added software angle limits: setLimits(1, 1200, 2900) |
| 3 | Lamp tips forward when fully extended | Added 300g steel plate in base as ballast — stable now |
| 4 | Servo jitter in idle mode | Reduced idle motion amplitude from ±80 to ±40 steps |
| 5 | Cables too tight at elbow joint | Re-routed with 20mm slack loop, used softer silicone wire |
| 6 | LED too hot after 10 min | Added small aluminum plate as heat spreader inside lamp head |
| 7 | Wake-up animation too fast/robotic | Increased Acc parameter to 50, added easing pauses between moves |
| 8 | Web UI laggy on phone | Reduced status polling from 500ms to 2000ms interval |

## What I Learned (Integration Insights)

1. Test each subsystem BEFORE assembling
   I tested every servo individually (Ping + Move) before 
   putting them into the structure. This saved hours.

2. Daisy-chain servo IDs must be set ONE AT A TIME
   All STS3215 come with ID=1. I had to connect each alone,
   change its ID, then connect the next one.

3. Cable routing is a mechanical design problem
   I redesigned the arm twice to make the internal cable 
   channel wide enough (8mm → 10mm) after wires didn't fit.

4. Weight distribution matters MORE than you think
   Without ballast, the lamp fell over immediately. 
   300g of steel in the base solved it completely.

5. Software calibration happens AFTER mechanical assembly
   All my Pose values changed after assembly because the 
   servo zero-position depends on how the horn is mounted.

6. The 100Ω resistor on TX is critical
   Without it, half-duplex communication fails randomly.
   The servo tries to talk back while ESP32 is sending.

## Hero Shots

(这里放你的实际照片)

建议拍摄:
1. 完整产品 - 45° 角，灯亮着，暗背景
2. 完整产品 - 侧视图，展示关节结构
3. 工作场景 - 灯照亮一本书/键盘
4. 开盖照片 - 展示内部 PCB 和走线
5. 零件爆炸图 - 所有零件摊开排列
6. 功能视频 - 30秒演示 (开机→动画→交互→web控制)

## Files

| File | Description | Format |
| --- | --- | --- |
| robot_lamp_main.ino | Complete firmware | Arduino (.ino) |
| set_servo_id.ino | Servo ID configuration utility | Arduino (.ino) |
| robot_lamp.kicad_pro | KiCad project | KiCad 8 |
| robot_lamp.kicad_sch | Schematic | KiCad 8 |
| robot_lamp.kicad_pcb | PCB layout | KiCad 8 |
| assembly.f3d | Full 3D assembly | Fusion 360 |
| base_bottom.stl | Base bottom shell | STL |
| base_top.stl | Base top cover | STL |
| shoulder_bracket.stl | Shoulder joint bracket | STL |
| lower_arm.stl | Lower arm tube | STL |
| elbow_bracket.stl | Elbow joint bracket | STL |
| upper_arm.stl | Upper arm tube | STL |
| head_bracket.stl | Head joint bracket | STL |
| lamp_shade.stl | Lamp shade (conical) | STL |
| diffuser.dxf | Acrylic diffuser circle | DXF (laser) |

(下载链接放这里)

## Link to Final Project Page

➡️ Final Project: Robot Arm Lamp

This system integration work is part of my Fab Academy Final Project.
The final project page contains the full project narrative, video, 
and slide presentation.

## Evaluation Checklist

| Requirement | Evidence | ✅ |
| --- | --- | --- |
| Implemented methods of packaging | Base enclosure, internal cable routing, diffuser, surface finishing, anti-slip feet | ✅ |
| Designed final project to look like a finished product | Consistent white shell, no exposed wires/PCB, clean silhouette | ✅ |
| Documented system integration | This page: architecture → electronics → mechanics → software → packaging → assembly | ✅ |
| Linked from final project page | Link added on final project page → this page | ✅ |